Experiment No 7

Objective: - Measurement of coefficient of discharge of Valve as function of valve lift.

TEST RIG SPECIFICATIONS:

The test rig contains, a centrifugal blower with venturi meter, Ball valve & Cylinder head with valves.

The valve opening is controlled by a nut & screw  mechanism.

Measurement of:-

- Valve geometry.

- Valve displacement.

- Flow rate through venturi & inlet pressure to the intake valve.

- Repeat the measurements for various flow rates.

Prerequisites: -

- Working principle of valves?

- What is a poppet valve?

- Name the geometrical parameters of a valve.

Experimental Procedure/what you should aim at: -

- Start the blower and adjust the blower ball valve to set the   flow rate.

- Fully open the engine intake valve.

- Measure the pressure difference across venturi.

- Also measure the inlet pressure to the intake.

- Repeat above steps for various valve positions.

- Repeat above steps for various blower flow rates.

- Obtain a relation/curve between valve lift & coefficient of discharge.

What next (Things you can ponder over):-

- Is there a smooth relation between Cd & Valve lift ?

- If not explain the nature of the relation?

- Identify the reasons for the discontinuous relation?

- Suggest a method to improve the design of valves?

Submission of Report:

- A single report per group.

- All are equally responsible for the preparation of the report.

- Should answer the questions on your report.

- Each Report will be evaluated for 10 points.

References:-(Books to consult)

- IC Engines by Heywood.

Theory:

Geometrical Description of a valve:

Stages of Valve Lift:

Instantaneous flow area:

Stage 1: Valve lift  Lv

Stage 2: Valve lift  Lv

Stage 3: Valve lift  Lv

Frictional Compressible Flow Through Inlet Valves

Where:

CD = Coefficient of discharge of the valve at a given lift.

pinlet : Inlet manifold pressure.

Tinlet: Inlet manifold temperature.

pcyl: Cylinder pressure.